# Introduction to Arduino

Objectives:

- To study the basics of an Arduino microcontroller.

- To learn how to program an Arduino to collect data and implement digital controls.

Equipment:

- Arduino Pro Mini Board

- FTDI Basic board – 5V

- USB cable: Type A connector – Mini-A connector

- PC computer

- Arduino software

- Resistors

- Light dependent resistors (LDR)

- Protoboard

References:

- Arduino Website: http://arduino.cc/en/

- Arduino Cookbook.  Author: Michael Margolis.  O'Reilly Media, ISBN-10: 0596802471, ISBN-13: 978-0596802479

Procedure:

      - The instructor will give an introduction to Arduino.

      - Connect an Arduino Pro Mini to a PC:

- On the FTDI board, find the pin labeled “GRN” and labeled “BLK”.

- Match the pins “GRN” and “BLK” on FTDI with the pins on Arduino Pro Mini.

- Connect the headers.

- Check the connection.

- Connect the USB cable to a PC and check if an on-board LED is on.

      - Open the Arduino integrated development environment and configure the board and port:

- Find the application named “Arduino IDE” and click it.

- In the sketch window,

          - Go to “Tools” → “Board”, select the correct board you are using: Arduino Pro or Pro Mini (5V, 16 MHz) w/ ATmega328.

          - Go to “Tools” → “Serial Port”.  Select the right port.  You may need to go to Device Manager on the PC to find out the port number.

      - Exercise 1: Blink an LED

- Go to “File” → “Examples” → “Basics” → “Blink”.

- Compile the program by clicking .

- Upload the program to the Arduino by clicking .

- Observe the blinking LED on the Arduino.

| Functions | Program for Exercise 1 |
| --- | --- |
| Setup(): initialization of Arduino - Pin Mode: Input or Output - Claim variables - Define constants Loop(): Main program runs continuously ;: end of a command pinMode(pin, value): Configure the specified pin to be either an input or an output. digitalWrite(pin, value): Write HIGH or LOW value to a digital I/O pin delay(value): Pauses the program for the amount of time (in milliseconds) specified as a parameter. | /* Exercise 1: Blink - Turns on an LED on for one second, then off for one second, repeatedly. */ void setup() { // initialize the digital pin as an output. // Pin 13 has an LED connected on Arduino board pinMode(13, OUTPUT); } void loop() { digitalWrite(13, HIGH); // set the LED on delay(1000); // wait for a second digitalWrite(13, LOW); // set the LED off delay(1000); // wait for a second } |

      - Exercise 2: Analog input and digital output with Arduino

- Connect the following circuit on the protoboard: Ground Vcc A00 LDR: Light-Dependent Resistor 100Ω - Enter the following program (Exercise 2-1). The program performs the following tasks: - Read the analog signal from the voltage divider, which is formed by a resistor and an LDR, on pin A0. - The voltage measured on pin A0 will change the blinking frequency of the internal LED. The duty cycle is maintained at 50%.

- Enter the next program (Exercise 2-2). This program maps analog readings between 200 and 800 into the blinking frequencies range of 5 Hz - 0.5 Hz.

// Exercise 2 – 1 const int ledPin = 13; // LED to D13 const int sensorPin = 0; // Sensor to A0 void setup() { pinMode(ledPin, OUTPUT); // enable output on the LED pin } void loop() { int rate = analogRead(sensorPin); // read the analog input digitalWrite(ledPin, HIGH); // set the LED on delay(rate); // wait duration dependent on light level digitalWrite(ledPin, LOW); // set LED off delay(rate); }

/* Exercise 2 – 2: Add a display on the rate of blinking */ const int ledPin = 13; // LED connected to digital pin 13 const int sensorPin = 0; // connect sensor to analog input 0 // the next two lines set the min and max delay between blinks const int minDuration = 100; // minimum wait between blinks const int maxDuration = 1000; // maximum wait between blinks void setup(){ pinMode(ledPin, OUTPUT); // enable output on the led pin Serial.begin(9600); // initialize Serial Monitor } void loop() { int rate = analogRead(sensorPin); // read the analog input // the next line scales the blink rate between the min and max values rate = map(rate, 200,800,minDuration, maxDuration); // convert to blink rate rate = constrain(rate, minDuration, maxDuration); // constrain the value Serial.println(rate); // print rate to serial monitor digitalWrite(ledPin, HIGH); // set the LED on delay(rate); // wait duration dependent on light level digitalWrite(ledPin, LOW); // set the LED off delay(rate); }

      - Exercise 3: Control A Servo Using A Potentiometer

- Connect the following circuit:

- Develop a sketch which can do the following:

i.	Read the voltage divider output signal at analog pin 0.

ii.	Control the position of the servo using the potentiometer.  The servo should turn from 0° to 180° as the potentiometer turns from one limit to the other.

iii.	Display the voltage and angle reading on a serial window.

/* Exercise 3 – 1: Control A Servo Using A Potentiometer */

#include <Servo.h>  //adds the Arduino Servo library to our code se we can easily control servos

Servo MyServo;

const int sensorPin = 0;          // connect sensor to analog input 0

const int MyServoPin = 9;     // connect the servo signal input to pin 9

int voltageIn;   // variable to store the voltage read

int servoAngle = 90; //servo angle variable, initialized to 90

const int minAngle = 0;     // minimum servo angle

const int maxAngle = 180;    // maximum servo angle

void setup(){

MyServo.attach(MyServoPin);   // initializes the servo control to be pin 8

Serial.begin(9600);                    // initialize Serial Monitor

MyServo.write(servoAngle);  //move the servo to the 90 degree position

delay(2000);  // added this in so you can see that the servo moves to the 90 degree position

}

void loop() {

voltageIn = analogRead(sensorPin); // read the analog input

servoAngle = map(voltageIn, 200,800,minAngle, maxAngle); // convert the voltage read to an angle

servoAngle = constrain(servoAngle, minAngle, maxAngle); // ensures that the angle won’t be greater than 180° or less than 0°.

MyServo.write(servoAngle); // change the angle of the servo

Serial.print(voltageIn);                          // print voltage to serial monitor

Serial.print(‘  ‘);	//print spaces to separate the voltage and the angle on the display

Serial.println(servoAngle);	//print the servo angle

}